# C Programming – Assignment 1

Grama Jyothi Rural Electricity Co-operative: A Reliable Billing Program

Name: Lingesh | Register Number: 1924260290

# Task 5 – Header Files and Conditional Compilation

Question: Explain header files, include guards, and conditional compilation in a modular electricity billing program.

Answer:

Header files contain declarations, function prototypes, type definitions, and constants shared across multiple source files.

A modular billing program can be divided into:

- billing.h – constants, types, and function prototypes.

- tariff.c – electricity consumption and bill calculation functions.

- billing_main.c – the main program and user interaction.

## Example: billing.h

#ifndef BILLING_H
#define BILLING_H

#define SLAB1_UNITS 100
#define SLAB2_UNITS 200

#define RATE1_PAISE 0
#define RATE2_PAISE 225
#define RATE3_PAISE 450

#define FIXED_CHARGE_PAISE 5000L

typedef unsigned short meter_t;

enum bill_status {
    BILL_OK = 0,
    ERR_RANGE,
    ERR_NO_ROLLOVER
};

unsigned int consumption(meter_t previous,
                         meter_t current);

long compute_bill(int units);

#endif

## Explanation

1. Include guards

#ifndef, #define, and #endif prevent the same header file from being processed repeatedly within a single translation unit.

2. Conditional compilation

Conditional compilation uses directives such as #ifdef and #ifndef to include or exclude source code during preprocessing.

Example:

#ifdef DEBUG
#define DBG(...) fprintf(stderr, __VA_ARGS__)
#else
#define DBG(...) ((void)0)
#endif

3. Header inclusion

- #include <stdio.h> searches the standard system include paths.

- #include "billing.h" normally searches the source file’s directory first.

## Compilation command

gcc -Wall -Wextra billing_main.c tariff.c -o billing

Conclusion: Header files improve modularity, reduce duplicated declarations, and make the billing program easier to maintain.