# C Programming – Assignment 1

Grama Jyothi Rural Electricity Co-operative: A Reliable Billing Program

Name: Lingesh | Register Number: 1924260290

# Task 8 – Error Handling and Validation

Question: Explain how the billing program should handle invalid meter readings and rollover conditions.

Answer:

A reliable billing system must reject invalid inputs before calculating the bill.

The program should check:

- Household number is positive.

- Meter readings lie between 0 and 65535.

- Input values are numeric.

- Consumption does not exceed the maximum believable monthly usage of 5000 units.

- A lower current reading is accepted only when it represents a valid meter rollover.

## C program

#include <stdio.h>

#define METER_MAX 65535U
#define MAX_UNITS 5000U

int get_units(long previous, long current,
              unsigned int *units)
{
    if (previous < 0 || previous > METER_MAX ||
        current < 0 || current > METER_MAX) {
        return 1;
    }

    *units = (unsigned short)(current - previous);

    if (*units > MAX_UNITS) {
        return 2;
    }

    return 0;
}

int main(void)
{
    unsigned int units;
    int status;

    status = get_units(1200, 1530, &units);

    if (status == 0) {
        printf("Units consumed: %u\n", units);
    } else {
        printf("Invalid readings or consumption.\n");
    }

    status = get_units(65400, 120, &units);

    if (status == 0) {
        printf("Rollover consumption: %u\n", units);
    } else {
        printf("Invalid readings or consumption.\n");
    }

    return 0;
}

## Output

Units consumed: 330
Rollover consumption: 256

## Explanation

A 16-bit meter wraps from 65535 to 0. Therefore:

Consumption=65536-65400+120=256

The maximum-consumption check helps distinguish a genuine rollover from an incorrect reading, subject to the assignment’s assumption of at most 5000 units per period.